# PRODUCT HEALTH AND SAFETY DATA SHEET

1.	PRODUCT NAME

Recycled paper products - Sugar Paper & Manilla

2.	HAZARD CLASSIFICATION

Non-hazardous

3.	COMPONENTS AND APPROXIMATE PROPORTIONS

Recycled fibre	up to					87%

Inorganic fillers (clay / chalk)	up to			18%

Sizing Agent (where added) up to			1.0%

Dyes (where added) up to				3.0%

Other additives up to					0.1%

4.	PH level

Typical value 7.5 to 8.0

5.	HEALTH HAZARD INFORMATION

The only route to exposure is inhalation of combustion products or dusts. With respect to inhalation of combustion products the casualty may require medical treatment for the effects of smoke inhalation.

Refer to HSE guidance note EH40 regarding occupational exposure limits of “cellulose Dusts”, and hardwood dust.  Corrugating and conversion processes will generate dust and, therefore, local exhaust ventilation may be required to comply with COSHH regulations on plant being used to carry out these or similar processes.

6.	HANDLING PRECAUTIONS

None required in the context of this document.

7.	FIRE HAZARDS

Paper and board products are non-flammable, but burn easily.  We strongly advise that a “No Smoking” rule is enforced in any process that involves paper and board products.

7.1.	Unusual Hazards

If improperly stored (i.e. damp conditions) paper and board can decompose, generating considerable heat.  There have been numerous cases of serious fires (mainly in waste paper stacks) due to spontaneous self-combustion.

7.2.	Combustion Products

Carbon (soot), Carbon Dioxide, Carbon Monoxide.

7.3.	Flash Point

Not applicable.

7.4	Auto Ignition Temperature

451 degrees Fahrenheit (232 degrees Celsius).

7.5.	Fire Extinguishing Media

Water, Carbon Dioxide.

8.	WASTE DISPOSAL

Significant quantities of waste should be sold to a merchant for further recycling.

9.	STORAGE

Store in a cool dry place.  (See unusual hazards above).

10.	EMERGENCY TELEPHONE NUMBER

Not applicable.

11.	ADDTIONAL INFORMATION

Refer to HSE guidance note EH40 regarding occupational exposure limits of “Dusts”.  Corrugating and conversion processes will generate dust and, therefore, local exhaust ventilation may be required to comply with COSHH regulations on plant being used to carry out these or similar processes.